Lab Practical #05:

Study the concept of VLAN using packet tracer.

Practical Assignment #05:

- Implement the different network structures in VLAN and VLAN trunking. Also check connectivity between them using ping command or PDU utility.

Steps to create VLANs in Packet Tracer:-

Step 1: Open Switch Configuration

- Double-click on the switch in Packet Tracer.

- Go to the Config tab.

Step 2: Create VLANs

- In the left panel, click on VLAN Database.

- In the VLAN section, add new VLANs by typing the VLAN ID and Name, then click Add.

  - Example:

    - VLAN ID: 10, Name: Sales

    - VLAN ID: 20, Name: HR

Step 3: Assign Ports to VLANs

- From the left panel, select each port (e.g., FastEthernet 0/1, 0/2, 0/3, 0/4).

- For each port:

  - Change Port Mode to Access.

  - Set VLAN Membership to the desired VLAN (e.g., VLAN 10 or VLAN 20).

- Example:

  - Ports Fa0/1 and Fa0/2 → VLAN 10 (Sales)

  - Ports Fa0/3 and Fa0/4 → VLAN 20 (HR)

Step 4: Verify VLANs

- Go back to VLAN Database.

- You should see the VLANs created (Sales and HR) and the ports assigned to them.

Step 5: Testing

- Connect PCs to the switch and assign IP addresses to them.

- PCs in the same VLAN will communicate with each other.

- PCs in different VLANs cannot communicate unless a router or Layer 3 switch is configured for Inter-VLAN Routing.

Instructions:

- Different VLANs configuration setup screenshot. (VLAN example given by lab faculty)

- Write steps to create VLANs in packet tracer.

- Mention IP address of each pc as label.

- Ping command or PDU screenshot between two VLANs.